SnowPALM Technical Documentation

Contents:

- Introduction ------------------------------------------------------------------------------------------	1

- Running SnowPALM --------------------------------------------------------------------------------	..

2.1) Model Design ----------------------------------------------------------------------------------	..

2.2) Required Software ----------------------------------------------------------------------------	..

2.3) Input Datasets ---------------------------------------------------------------------------------	..

2.4) Processing Steps -------------------------------------------------------------------------------	..

- Model Process Description -----------------------------------------------------------------------	..

3.1)   Precipitation and Humidity ---------------------------------------------------------------	..

3.2)   Radiation and Albedo ----------------------------------------------------------------------	..

3.2.1)   Shortwave Radiation -------------------------------------------------------------	..

3.2.2)   Longwave Radiation --------------------------------------------------------------	..

3.2.3)   Net Radiation and Albedo -------------------------------------------------------	..

3.3)   Snowpack Mass and Energy Balance ---------------------------------------------------	..

3.3.1)   Sensible and Latent Heat Flux --------------------------------------------------	..

3.3.2)   Heat from Precipitation ----------------------------------------------------------	..

3.3.3)   Ground Heat Flux ------------------------------------------------------------------	..

3.3.4)   Melt -----------------------------------------------------------------------------------	..

3.3.5)   Longwave Radiation --------------------------------------------------------------	..

3.3.6)   Net Radiation and Albedo -------------------------------------------------------	..

3.4)   Canopy Interception-------------------------------------------------------------------------	..

3.4.1)    Snowfall -----------------------------------------------------------------------------	..

3.4.2)    Rainfall -------------------------------------------------------------------------------	..

3.5)   Wind Distribution of Snowfall ------------------------------------------------------------	..

3.6)   Snow Density ---------------------------------------------------------------------------------	..

3.7)   Near Surface Hydrology Model ----------------------------------------------------------	..

- Model Parameters ----------------------------------------------------------------------------------	..

- Explanation of Model Codes ---------------------------------------------------------------------	..

- References --------------------------------------------------------------------------------------------	..

1) Introduction

SnowPALM is a high resolution snow model for simulating the effects of forest cover and its change on snowpack.  It can run at variable temporal (hourly or daily) and spatial resolutions, but is most commonly implemented using lidar data at 1 m resolution.  SnowPALM's internal model physics allows it to represent complex patterns of snow variability typical of forested sites. Notably, it predicts that areas that do not have overhead canopy, but are still influenced by nearby canopy, have more snow variability than areas that are far from canopy (Broxton et al., 2015). This is different from many Land Surface Models, which use a binary classification of either open or under-canopy, limiting their ability to reproduce lidar observations of snow or handle different spatial configurations of forest canopy structure. SnowPALM explicitly represents three-dimensional influences that trees (and other topographic features) have on the snowpack (e.g., how they modify shortwave radiation, longwave radiation, and wind distribution of snowfall).

2) Running SnowPALM

2.1) Model Design

2.2) Required Software

2.3) Input Datasets

2.4) Processing Steps

3) Model Process Description

SnowPALM is an energy balance snow model that calculates snow states (SWE, snow depth, cold content), snowpack mass fluxes, and energy fluxes to and from the snowpack.  The model combines a variety of formulations for canopy-affected snow processes such as interception, canopy attenuation of radiation, and wind distribution of snowfall. There are separate layers representing the snowpack and soil, and a thin surface layer to compute radiative and turbulent exchanges with the atmosphere. The model uses as input hourly values of air temperature, precipitation (which is partitioned into rainfall and snowfall based on a calibrated air temperature threshold), humidity, pressure, and incoming shortwave and longwave radiation. Shortwave radiation is adjusted for inclined slopes and direct/diffuse components. Direct and diffuse components of shortwave radiation, as well as longwave radiation, are attenuated due to canopy influences using Beer's Law (Mahat & Tarboton, 2012). SnowPALM also computes shading caused by nearby vegetation and topography based on sun angle for every hour, and longwave radiative effects are computed based on the temperature of nearby canopy and an empirical scaling factor that decays with distance to canopy. Interception is simulated using the model of Liston and Elder (2006), which accounts for interception, sublimation of intercepted snow, and snow unloading/drip. Wind distribution of snowfall is simulated using spatial indices (Winstral & Marks, 2002), which are based on the height of nearby upwind terrain and vegetation features.  Below is a comprehensive set of model formulations for various processes in SnowPALM.

3.1) Precipitation and Humidity

SnowPALM can either incorporate already partitioned rainfall and snowfall, or it has algorithms to separate the rainfall and snowfall based on air temperature.  If required, rainfall-snowfall partitioning is accomplished using a temperature range, the lower limit of which is all snowfall, and the upper limit of which is all rainfall.  We use a continuous function to model this transition:

fs=max⁡0,min⁡1,1-dhdxTa-Ta,min-dh2πsin2πdxTa-Ta,min     (1)

In this expression, dh=1, dx=Ta,max-Ta,min, Ta,max=Ta+Ta,range/2, and Ta,min=Ta-Ta,range/2. T, the air temperature threshold, and Ta,range, the range over which the rain-snow transition occurs, are user defined parameters.

In addition to the rain-snow transition, there are also expressions to compute vapor pressure, saturated vapor pressure, and ultimately, relative humidity (the ratio between the two).

Vapor pressure,  e, is computed from the daily dewpoint temperature, Td, using Teten’s formula (Monteith and Unsworth, 2008):

e=0.6108×exp⁡17.27×Td237.3+ Td×1000     (2)

Similarly, saturated vapor pressure, esat, is computed from the air temperature, T:

esat=0.6108×exp⁡17.27×Ta237.3+ Ta×1000     (3)

3.2) Radiation and Albedo

3.2.1) Shortwave Radiation

Solar Radiation is given as a forcing input.  However, SnowPALM also computes potential solar radiation for a particular location, including on inclined surfaces.  This potential radiation is used to adjust the given solar radiation values for slope and aspect influences by multiplying the given solar radiation values by the solar forcing index computed for the hillslope, where

SFI=RsR0 	     (4)

Rs is the potential solar radiation on the inclined surface and R0 is the flat surface potential solar radiation.

The potential solar radiation (both R0and Rs) is computed using a computer code developed by Dr. Felix Hebeler, dept. of Geography, University of Zurich, slightly modified to accept the slope-aspect conventions in SnowPALM (and converted from Matlab to Python).  The code follows the approach of Kumar et al. 1997 (note that specific equations related to the algorithm can be found in that reference). It calculates clear sky radiation corrected for the incident angle (self-shading) plus diffuse and reflected radiation.

There are also a user-adjustable parameter that applies a simple multiplier to the shortwave radiation (after any adjustments are made for slope and aspect) separately for winter and summer to account for potential biases during different times of year in the forcing dataset (e.g. if NLDAS data is used) so the final downward shortwave radiation is computed is:

Rsd= Robs×SFI×fr     (5)

where Robs is the observed solar radiation, SFI, is the solar forcing index (from 4) and fr is a potential seasonally variable user defined multiplier.

The canopy also causes attenuation of incoming solar radiation. For radiation attenuation, we use the model described by Mahat and Tarboton (2012) for canopy radiation transmission. This model is ideal to use on SnowPALM's distributed model grid because it assumes multiple scattering of the light beams, it is more parsimonious and computationally efficient than detailed multilayer radiative transfer models and it uses LAI as the key model parameter. Direct canopy transmittances are given by (Mahat and Tarboton, 2012)

τb=exp⁡-k'KbLF (6)

Where k'=1-β (β is a coefficient that quantifies the effect of multiple scattering),
Kb=Gcos⁡(ϑ) is the non-scattering attenuation coefficient (G is the geometry factor, and θ is the solar zenith angle), and LF is effectively a canopy level leaf area index.  Following Mahat and Tarboton (2012), both β and G are assumed to be 0·5, and here LF is given as a user defined parameter.  Diffuse canopy transmittances are given by:

τb=(1-k'GLF)e-k'GLF+(k'GLF)2Ei(1,k'GLF) (7)

where Ei connotes the exponential integral.  Because radiation attenuation occurs not only beneath but also beside tree canopies, SnowPALM first calculates pixels that are shadowed given the vegetation surface (which is the LiDAR-derived ground surface + vegetation height for each 1-m pixel), the horizon angle and azimuth of the sun (using the solar analyst in SAGA GIS). Then, once the locations of the shadows are determined for each time step, Beer's law attenuation factor is applied for all pixels that are in shadow, regardless of whether they are underneath or next to trees.

3.2.2) Longwave Radiation

At the snowpack, a sky-view factor (which is precomputed based on SAGA GIS algorithms) is used to weight longwave radiation emitted from an unobstructed sky (given by the forcing data) and longwave radiation emitted in a forested setting:

R=SVF×Rld+1-SVF×τl×Rld+1-τl×Rc (8)

where Rlds is the longwave radiation above canopy, SVF is the sky-view factor, Rld is the above-canopy incoming longwave radiation, εc is the canopy emissivity, σ is the stefan-boltzman constant and τl is the canopy transmittance for longwave radiation (assumed to be the same as that for diffuse shortwave radiation, given by equation 7 above). Radiation emission by the canopy is computed by Rc=εcσTc4, where εc is the canopy emissivity, σ is the Setfan–Boltzmann constant, and Tc is the skin temperature of the canopy in K.

There is also an empirical enhancement to longwave radiation due to woody parts of plants heating up in the sun.  In reality, this sort of enhancement occurs when tree trunks are exposed to direct sunlight and heat up significantly more than the rest of the canopy.  In SnowPALM, this is simulated by finding regions where the top-of-canopy solar forcing index when the canopy height is close to the ground is greater than the below canopy solar forcing index, and then applying smoothing factor to account for the fact that the enhancement not only occurs within the 1-m pixels where this condition is met, but also for surrounding pixels as well.  Algorithmically, this is represented by applying spatial smoothing to the following:

LWI=a*(max0, 1-Vhhmax×max0, SFITop-SFIBC (8)

where LWI is a map of longwave enhancement, Vh is a map of canopy height, hmax is a parameter representing the maximum height of canopy that causes this type of enhancement (where smaller values limit enhancement to areas where sun strikes trees closer to the ground),  SFITop is the top of canopy solar forcing, and SFIBC is below canopy solar forcing and a is a parameter representing the efficiency of this LWI enhancement.  This procedure highlights areas that are on the north sides of medium-large clearings where there southern winter sun is not blocked by trees and can directly warm the tree trunks.  Note these LWI maps are created during the preprocessing stage, and it is up to the user how often LWI maps are created (e.g. hourly, daily or monthly).  If they aren’t created for every model timestep (e.g. hourly or daily), LWI for each map pixel is temporally interpolated between times when they are created (since there is small incremental variation in this field for each new day).

3.2.3) Net Radiation and Albedo

Net Radiation is computed as:

Rn=Rsd1-α+Rld-Rlu     (9)

where Rsd is the incoming solar radiation, Rld is the incoming longwave, Rlu is the outgoing longwave radiation, and α is the snow albedo.

Rlu is calculated using the Stephan-boltzmann equation:

Rlu=εsσTs4      (10)

where εs is the snow emissivity (0.99), σ is the Stefan-Boltzmann constant (5.67×10-8Wm2K), and Ts is the snowpack temperature.

Snow albedo, α, is computed as a function of the age of the snow surface (since the last significant snowfall):

α=αs,max-αs,min exp-τk+αmin      (11)

αs,max  is the albedo of fresh snowfall, αs,min is the albedo of old snow, τ is the age of the snow surface, and k is a decay constant.  Every day, τ increments by one day, but on days that it snows, τ is adjusted (to 0 if the storm is large enough) using:

τ= τ0max⁡0,ar-Sar      (12)

Where S is the snowfall, ar is a parameter specifying the snowfall size that will completely reset the snow surface albedo.  αi, αmin, k, and ar are user defined parameters.

3.3) Snowpack Mass and Energy Balance

The mass balance of the snowpack is:

dSWEdt=Ps+Pr-E-M      (13)

where Ps is snowfall (which includes both throughfall and unloaded snow, if under canopy), Pr is rainfall (which includes both rainfall and drip from the canopy), E is sublimation, and M is snowmelt.

The energy balance is:

dccdt=Qn+Qh+Qe+Qg+Qm      (14)

Where cold content is limited so that the bulk snowpack temperature is between the snow surface and 0oC.

3.3.1) Sensible and Latent Heat Flux

Sensible heat flux when the temperature is computed as:

Qh=KhρacpTs-Ta    (15)

Here,  cp is the air specific heat capacity (1005 Jkg°K), Kh is the sensible heat conductance, ρa is the air density, Ta is the air temperature, and Ts is the snow temperature.

The variation of ρa, the air density, with elevation is estimated because air pressure is not given.  ρa is calculated as:

ρa=PRdTa        (16)

Where pressure is estimated to be:

P=P0×exp⁡-gzRdT0       (17)

Here, Rd, the specific gas constant for dry air, is 287.058 Jkg°K,  g, the acceleration due to gravity, is 9.81 ms2, and T0=Ta+L×z, L is an estimated lapse rate (6.5 °Kkm), P0 is standard sea level pressure (101.325 kPa) and z is the elevation.

The latent heat flux is computed as:

Qe=Ke0.622×hvRd×Taea-es(Ts)     (18)

where Ke is the latent heat conductance, es(Ts) is the saturated vapor pressure at the snow surface, ea is the air vapor pressure, Rd is the dry gas constant, hv is the latent heat of sublimation (2834 kJ/kg).

The water equivalence depth of sublimation is:

E=Qe×dtρwhv     (19)

ρw is the density of water (1000 kg/m3), and dt is the model timestep.

The heat conductance terms are given by expressions of turbulent transfer in the boundary layer (Anderson, 1976; Male and Gray, 1981; Brutsaert, 1982).  For neutral conditions:

Kneutral=Kh=Ke=k2Vln⁡zz02     (20)

Here, k is von karman’s constant (0.4), V is the wind speed at measurement height, z, and z0 is roughness height at which the logarithmic boundary layer profile predicts zero velocity.  Both z and z0 can be specified as model parameters.  A stability correction is applied based on the Richardson number, which is approximated as:

Ri≈gTa-TszV2Ta       (21)

Ri is used to adjust Kh and Ke following Price and Dunne (1976):

Kadj=Kneutral1+10Ri, Ri≥0, Stable Conditions     (22)

Kadj=Kneutral1+10Ri, Ri<0, Unstable Conditions  (23)

As in Tarboton and Luce, 1996, there is also a stability factor to control the extent that these correction factors are applied:

Kh=Ke=Kneutral+FstabKadj-Kneutral    (24)

Fstab is a user defined parameter ranging from 0-1 (0 uses neutral conductances and 1 gives full stability corrections).

3.3.2) Heat from Precipitation

Heat from precipitation considers heat from rainfall and snowfall separately (assuming that rainfall temperature is the maximum of air temperature and freezing temperature, and snowfall temperature is the minimum of air temperature and freezing temperature):

Qp=PsCsρwminTa,0+Prhfρw+CwρwmaxTa,0     (25)

Ps is the snow rate, Pr is the rain rate, Cs is the specific heat of ice (2.05 kJkg-K), Cw is the specific heat of water (4.181 kJkg-K), ρw is the density of water (1000kgm3), and hf is the latent heat of fusion (334 kJkg-K).

3.3.3) Ground Heat Flux

Ground heat flux is given by:

Qg=-kdTdz    (26)

k is the soil (and/or snow) thermal conductivity, dz is the distance between the bottom and top of the modeled layer (e.g. the snowpack surface, middle of the snowpack, snow-soil interface, or damping depth), and dT is the temperature across this vertical difference.

3.3.4) Melt

SnowPALM computes potential melt energy based on energy contributions of other energy balance terms:

Qm×dt=-max⁡(0,cc+Qn+Qh+Qe+Qp+Qg×dt)    (27)

Here, Qn is the net radiation, Qh is the sensible heat, Qe is the latent heat, Qp is the precip heat, Qg is the ground heat, dt is the model timestep, and cc is the cold content.

In this expression, SWE is the snow water equivalent, ρw is the density of water (1000kgm3), Ci is the specific heat of ice (2050 Jkg-K), and Tp is the lesser of the average of the last n days temperatures and 0oC (where n is a user-defined parameter).

Melt mass flux is computed as:

M=-Qm×dtρw×hf       (28)

Melt is limited such that it cannot exceed SWE (note that the energy flux is also limited).

3.4) Canopy Interception

3.4.1) Snowfall

Canopy interception of snowfall follows Liston and Elder, 2006 and Pomeroy et al., 1998:

It=It-1+0.7Imax-It-11-exp⁡SImax-u-e-d   (29)

where It is the intercepted snow for the current timestep, It-1 is the intercepted snow for the previous timestep, S is the snowfall, Imax is the maximum interception capacity, given by u is the snow unloading rate, e is the canopy sublimation rate, and d is the melt-drip rate.  The maximum interception capacity is given by:

Imax=Cs×LAI    (30)

where LAI is the total leaf area index (that includes stems, leaves, and branches) and Cs is a user defined multiplier.  The amount of snow unloading  (u) is estimated as a function of the amount of intercepted snow:

u=pu×I   (31)

where I is intercepted snowfall (after the addition of new snowfall, but before subtractions from sublimation and drip), and pu is a user definable parameter specifying snow unloading rate.  The amount of sublimation of intercepted snowfall (e) is computed as in Liston and Elder (2006).  These equations are numerous and are not reproduced here but can be found in section 3 of Liston and elder (2006).  There is also a user defined multiplier that can adjust the canopy sublimation rate:

e=pe×e0      (32)

where e0 is the canopy sublimation given by the above methodology, and pe is the user-defined multiplier.  Finally, the melt drip rate is given by a temperature index formulation:

d=max⁡(0,pd×T)      (33)

where T is the temperature (in celcius), and pd is a user defined parameter representing the melt drip rate.

3.4.2) Rainfall

Interception of rainfall is treated similarly to the interception of snowfall except there is no unloading term, and canopy evaporation is tied to potential evaporation with a user definable parameter, and drip of intercepted rainfall has no temperature dependence.  The canopy mass balance for intercepted rainfall is given by:

It=It-1+0.7Imax-It-11-exp⁡RImax-e-d (34)

Where the model terms are analogous to those for equation 34-38, except using a separate set of parameters for rainfall.

3.5) Wind distribution of snowfall

In SnowPALM, wind distribution of snowfall is treated as an empirical function of shelter and exposure. Users can choose between two potential wind models based on exposure to wind: Boehner et al 2009 and Winstral et al., 2002.  The Boehner et al. (2009) approach, estimates terrain exposure and sheltering by analyzing the relationship between airflow direction and surrounding topography using digital elevation models (DEMs). The resulting dimensionless index identifies wind-exposed ridges and crests (values > 1) as well as sheltered leeward areas (values < 1).  The Winstral et al. (2002) model, calculates shelter or exposure by evaluating the maximum upwind slope angle within a specified search distance and directional sector, effectively quantifying how terrain obstructs or channels wind flow.

Both the Boehner et al 2009 and Winstral et al., 2002 models are implemented in Saga GIS and computed for all possible wind angles as a preprocessing step.  Then, wind distribution of snowfall is computed as an empirical function of the spatial fields generated by these models.  On each day with potential snowfall (precipitation with an air temperature below a threshold temperature (e.g. 10C), the distribution map is chosen based on the prevailing wind direction for that day (or alternatively, for a single user defined direction).  Wind distribution maps are computed based on existing canopy cover as well as one based on bare-earth topography.  There is a user defined parameter indicating whether the final wind distribution factor should based solely on the bare earth wind model, the one with full canopy cover, or should be a mixture of the two.

3.6) Snow Density

Snow density in SnowPALM is given by:

ρt=max⁡ρi,min⁡ρmax,1-f×ρt-1+f×ρi+ds+ds+dl          (35)

Here, ρt is the snow density for the current timestep, ρt-1 is the snow density for the previous timestep, ρi is a user-definable parameter representing the density of new snowfall,  ρmax is the maximum allowable snow density, f is the fraction of pack that is contributed by new snowfall (f=dndt, where dn is the new snow depth, and dt is the total snow depth (including new snowfall). ds is the densification due to overburden, which is parameterized as:

ds=ρi+SWEρmax-ρps     (36)

where SWE is the snow water equivalent, ρ is the snow density (after the addition of new snow) and  ps is a user-definable parameter specifying densification rate due to overburden.  dw is the densification due to wind effects, which is parameterized here as:

dw=wρmax-ρpw   (37)

where w is the wind speed, and pw is a user-definable parameter specifying densification rate due to wind.  dl is the densification due to liquid water in the pack, and is parameterized here as:

dl=rρmax-ρpl    (38)

where r is a flag specifying whether the snowpack is ripe (1 if ripe, and 0 if not ripe), and pl is a user-definable parameter specifying densification rate due to liquid in the snowpack.

3.6) Near Surface Hydrology Model

After the rainfall and snowmelt are combined, it is infiltrated into the soil.  Infiltration runoff is computed as

Ri=NWI×Fm×(max⁡(0,SM-SMi,min) /(SMi,max-SMi,min)   (39)

where NWI is the net water input, Fm is the maximum possible fraction of infiltration excess runoff (e.g. at saturated conditions), SM is the soil moisture content, SMi,max is the soil moisture at which infiltration excess runoff is maximized.

Next, evapotranspiration is computed as:

ET=PET×min⁡(1,max0,SM-WP/(CMC-WP)     (40)

where CMC and WP are, respectively, critical moisture content, and wilting point.  PET is given as a forcing input.

Percolation out of the top soil layer is computed as:

D=k×SM-SMresSMsat-SMres2b+3  (41)

where k is the saturated hydraulic conductivity, SM is the soil moisture, SMsat is the saturated soil moisture, SMres is the residual soil moisture, and b is the pore size distribution index.  As in Kineros2, the soil specific residual soil moisture content is computed as an empirical function of the soil capillary drive parameter,

SMres=0.1+0.31+1g/100040.25   (42)

Capillary rise, itself, is computed as:

cr=bwkαψa/H2×b/3      (48)

where bw is SM-SMresSMsat-SMres, k is the saturated hydraulic conductivity, α=1+(3/2)/((2×b3)-1) ψa is the air entry pressure, H is the thickness of the upper soil layer, and b is the pore size distribution index.

Below the top soil layer, there are two conceptual layers representing 1) a shallow aquifer in the vadose zone (called the vadose zone aquifer), and 2) discharge from groundwater (called the phreatic zone aquifer), which enables computation of baseflow originating from the deep soil layers.

Discharge from both reservoirs is computed as:

q=aSb      (43)

where S is the storage in each reservoir, and a and b, are constants.  There is also a leakage term between the two reservoirs, which is simply computed as

ql=cSvadose      (44)

where Svadose is the storage of the vadose zone aquifer, and c is a constant.

In addition to simulating water flow into and out of the top soil layer, SnowPALM also simulates heat conduction through this layer, as well as soil freezing / thawing.  Temperature is computed similarly to the snowpack, where energy can be added / removed from the soil layer from above or below:

Qsoil,t=Qsoil,t-1+Qtop+Qbottom      (45)

where Qtop and Qbottom are heat fluxes into and out of the top and bottom of the soil layer are computed using equation 29 (where the surface / snowpack temperature are used to compute the flux at the top, and a constant temperature and a damping depth are used to compute the flux at the bottom.

When the soil temperature is above or below freezing, the energy is used to warm and cool the soil temperature, where:

Tsoil=Qsoilρi×SMi×Ci+ρw×SMw×Cw+ρs×(H-SMi-SMw)×Cs   (46)

where Qsoil is the energy content of the soil layer, SMi is the mass of ice in the soil layer, SMw is the amount of liquid water in the soil layer, H is the soil layer thickness, Ci, Cw, and Cs are the specific heats of ice, liquid water, and soil, and ρw, ρw, and ρs are the densities of ice, water, and dry soil.

When the temperature drops to freezing or rises to freezing, the energy is used to drive soil melt / freezing.  This is calculated using equation 31, except Qm is the cold content of the soil layer, and the amount of ice produced is limited between 0 (unfrozen condition) and the soil moisture content (all water is frozen), and excess energy is used to raise and lower the soil temperature using equation 52.

4) Model parameters

model_pars = {

# Forcing Data Adjustment

'snow_mult': 1.1,               # Snowfall Multiplier [-]

'srad_mult_summer': 1.05,       # Shortwave Radiation Multiplier (summer) [-]

'srad_mult_winter': 0.9,      	# Shortwave Radiation Multiplier (winter) [-]

'lrad_mult_summer': 1.1,        # Longwave Radiation Multiplier (summer) [-]

'lrad_mult_winter': 1.1,      	# Longwave Radiation Multiplier (winter) [-]

'airt_adj': 0,                  # Adjustment to air temperature [C]

'rh_mult': 1.0,                 # Relative humitidy multiplier [-]

'RainThresh': 2,                # Rain-Snow transition temperature [C] (suggested range: -1 - 3, only if snowfall is not perscribed)

'RainThresh_dh': 1.0,           # Temperature range over which the rain/snow transition occurs [C] (suggested range: 1 - 2)

'diffuse_fr_0': 0.1,            # Fraction of shortwave radiation that is diffuse [-] (suggested range: 0.1 - 0.15)

'LF_trans': 2,                  # Effective LAI for computing for direct and diffuse shortwave radiation under canopy  for Mahat and Tarboton model [-] (suggested range: 2 - 4)

'lwi_par': 3.15,                # LWI (Or longwave influence of warm tree trunks at the edges of canopy stands) Parameter 1 (suggested range: 0.5 - 1.5)

'snowfall_wind_trans': 0.0,     # 0 -> Only below canopy wind index is used, 1-> Only above canopy wind index is used), or can be a mixture (suggested range: 0 - 0.5)

'snowfall_wind_exp': 2,         # Exponent applied to wind index map (suggested range: 0.5 - 2), only if wind model is on

# Higher values accentuate wind distribution effect, lower values mute wind distribution effect

# Albedo Parameters

'NoSnowAlbedo_winter': 0.22,    # Albedo of bare ground during the winter (suggested range: 0.2 - 0.3)

'NoSnowAlbedo_summer': 0.14,    # Albedo of bare ground during the summer (suggested range: 0.1 - 0.2)

'Snowfall_factor': 200,         # Size of snowfall to fully reset albedo [mm] (suggested range: 10 - 50)

'MaxAlbedo': 0.82,              # Maximum (or fresh snow) albedo (-) (suggested range: 0.8 - 0.9)

'minalbedo': 0.2,               # Minimum snowpack albedo [-] (suggested range: 0.3 - 0.4)

'albedo_decay_cold': 0.01,      # Albedo Decay Factor for cold snowpack [1/day] (suggested range: 0.01 - 0.05)

'albedo_decay_warm': 0.05,      # Albedo Decay Factor for cold snowpack [1/day] (suggested range: 0.05 - 0.15)

'albedo_enh_red': 0.5,          # Albedo enhancement or reduction based on SFI (0: no enhancement / reduction -> higher number: more enhancement / reduction)

# Snow Density Parameters

'density_min': 0.07,             # Density of new snow [g/cm3] (suggested range: 0.05-0.15)

'density_max': 0.5,             # Maximum snow density [g/cm3] (suggested range: 0.5-0.6)

'apar': 0.02,                   # Snow densification rate due to age [Fraction / day] (suggested range: 0.01-0.02)

'dpar': 0.002,                  # Snow densfication rate due to overburdin [Fraction / cm SWE] (suggested range: 0.001-0.002)

'rpar': 0.05,                   # Snow densfication rate due to liquid in snowpack [Fraction / day when isothermal snowpack] (suggested range: 0.01-0.1)

# Snow Interception Parameters

'melt_drip_par': 0.01,           # Melt Drip Rate [mm/day per deg-C above freezing] (suggested range: 0-1)

'snow_unload_par': 0.05,         # Fraction of canopy snow that unloads each day [-] (suggested range: 0-1)

'canopy_sub_mult': 2.0,          # Canopy sublimation multiplier applied to potential sublimation rate (which is computed for snowpack surface) [-] (suggested range: 5-20)

'C_S_cap_mult': 20,              # Factor multiplied by LAI to compute canopy snow storage capacity (suggested range: 5-20)

# Rainfall Interception Parameters

'inter_cap_mult': 5,            # Factor multiplied by LAI to compute canopy snow storage capacity (suggested range: 2-10) in mm

'drip_par': 5,                  # Drip rate of intercepted water (suggested range: 1-5 mm/day)

'canevappar': 1,                # Factor multiplied by PET to get canopy evaporation (suggested range: 1-5)

# Miscellaneous Snowpack Parameters

'max_sub_pct_when_snow': 0.1,   # Maximum fraction of snowfall that can be sublimation when snowing (suggested range: 0-0.5)

'max_melt_pct_when_snow': 0.1,  # Maximum fraction of snowfall that can be melt when snowing (suggested range: 0-0.5)

'Ch': 0.5,                      # Multiplier applied to sensible heating equation (suggested range: 0.5-1)

'ground_sub_mult': 0.5,         # Ground Sublimation Multiplier (suggested range: 0.5-1)

'sroughness': 1E-5,             # Snow surface roughness length [m] (suggested range: 1E-5 - 1E-4)

'windlevel': 10,                # Height of windspeed measurement [m]

'kappa_snow': 0.1,              # Soil thermal conductivity [W/m/K] (suggested range: 0.05-0.2)

'kappa_soil': 0.5,              # Snow thermal conductivity [W/m/K] (suggested range: 0.2-1)

'tempdampdepth': 2,             # Temperature at damping depth underneath a snowpack [C]

'dampdepth': 1,                 # Damping depth [m]

'groundveght': 0.2,             # Ground Vegetation height [m] (suggested range 0.05-0.2)

'fstab': 0,                     # Stability correction [0 1]

# Note: The soil / vadose zone model still needs some updates.  Right now, it is experimental

# Soil Layer Parameters

'PET_Mult': 0.3,                # Potential evapotranspirataion multiplier [-]

'max_infil_mult': 0.2,          # Fraction of incoming water that becomes infiltration excess when the soil moisture is above the level specified by sm_max_infil [-]

'sm_max_infil': 0.2,            # Soil moisture content below which infiltration excess runoff is minimized [-]

'H': 300,                       # Thickness of surface soil layer [mm]

'ssat': 0.451,                  # Saturated water content [-] (current value for loam)

'sres': 0.05,                   # Residual water content [-] (current value for loam)

'psi_s': 146,                   # Soil air entry pressure [cm] (current value for loam)

'b_soil': 5.39,                 # Pore size distribution index [-] (current value for loam)

'k_soil': 600.48,               # Saturated hydraulic conductivity [mm/day] (current value for loam)

'wp': 0.05,                     # Wilting Point [-] (suggested range 0.05-0.15)

'cmc': 0.3,                     # Critical Moisture Content [-] (suggested range: 0.25-0.35)

# Vadose Zone Parameters

'coef_vadose': 0.1,             # Vadose Zone Reservoir Decay Parameter (multiplier) [-]

'coef_vadose2phreatic': 0.02,   # Leakage Rate between Vadose and Phreatic Reservoirs [-]

'coef_phreatic': 0.001,         # Phreatic Zone Decay Parameter (multiplier) [-]

# Initial States

'albedosnow_i': 0.5,            # Initial snow albedo [-]

'swe_i': 0,                     # Initial snow water equivalent [mm]

'cansnowstor_i': 0,             # Initial canopy intercepted snow storage [mm]

'canrainstor_i': 0,             # Initial canopy intercepted rain storage [mm]

'density_i': 0.1,               # Initial snow density [g/cm3]

'Tm_i': 0,                      # Initial snowpack temperature [C]

'T_soil_i': 0,                  # Initial top soil layer temperature [C]

'ice_percent_soil_i': 0,        # Initial soil ice fraction [cm3/cm3]

'sm_i': 0.3,                    # Initial soil moisture [cm3/cm3]

'q_vadose_i': 0.5,              # Initial water content in the vadose zone (below the top soil layer) [mm]

'q_phreatic_i': 0.1,            # Initial water content in the preatic zone [mm]

}

5) Explanation of Model Codes

6) References